Genesys Driver:  VB6 and RS-232

# Introduction

This is a description of a Visual Basic 6.0 driver software to be used with the TDK-Lambda Genesys DC power supply.

The driver is meant to make easy and reliable communication over the RS-232/485 bus.

The document applies to driver in file:      GEN RS VB Driver v04.zip

# Importing the Driver

## ZIP file

The delivered ZIP file has the following contents.  These are the three VB6 project files and two documentation:

## Driver Function Installation

The driver is a single function which may be copied directly into your VB6 application.

The function is found in file:   GenRsForm.frm

This function may return compiler errors in your application.

It contains a few variables that are declared globally outside of the function.  These variables  are used for debugging and for the example program.  They may be deleted.

## Example Program

The driver function is in an example VB6 project. This project sends a continuous loop of RS-232 commands to three Genesys PU.

The example RS-232 commands are the same as given by a customer in a VB6 project:

PU_lppo.vbp

However,  this “GenRs.vbp” driver example has fewer features than the customer’s example.

TO RUN THE EXAMPLE PROJECT:

- Use three Genesys PU in an RS-232/485 chain.

- Set RS-232 addresses to 1, 2, 3.

- Use RS-232 setting:  19200 Baud.

- Use port COM4 (may be changed in program).

EXAMPLE PROGRAM IS RUNNING:

Commands is the number of RS-232 commands sent in a continuous loop (until the Stop button is clicked).

Retrys Sent is the number of times a command had to be sent more than once to try and get a proper response.

# Using the Driver

The driver is used to send any RS-232 command (or query) to the Genesys PU.

It is a local function call.  Example:

status = PuSend(sendStr, res)

Where:

sendStr = 	RS-232 command string to send.
Examples:   ADR 3,  PV 5.22,  MC?,  REV?

status = 	Status after sending command.  Is Boolean.
TRUE = SUCCESS.  Command and response are acceptable.
FALSE = ERROR.  Improper response or no response
             after trying to send command four times.

res =	Result string (or response string).

If no error,  this is the message which the PU sent back after the command.  Typical examples may be “OK” or “5.220”

If error cannot be overcome,  this is the error returned by the PU or by the driver.  Details are given in next section.

An example driver call from the “GenRs.vbp” example project,

- The command “OUT 1” is sent to the PU in the “PuSend” function.

- If no error (status = SUCCESS),  then response,  “OK” is returned in “res” and the program continues to the next command (not shown)

- If an error occurred that could not be fixed by sending command four times (status = ERROR),  then a message box pops up and shows the command sent and what the error is.

- The error message box has two buttons.  
If OK button is clicked, the program goes to the next step (not shown).
If CANCEL button is clicked, the program ends.

# Driver Error Responses

If the driver function returns with “status = ERROR”,  then the parameter string “res” will have one of the following string values.

Driver Errors         (created when driver cannot get good response)

TMO	Timeout. No response from PU after sending command four times
CHKSUM	Checksum.  Response from PU damaged,  checksum not match.
CLOSED	The computer serial port is closed,  cannot send command.

Command Errors    (see Genesys User Manual for better description)

C01	Illegal command.
C02	Missing parameter.
C03	Illegal parameter.
C04	Checksum error.
C05	Setting out of range.

Execution Errors    (see Genesys User Manual for better description)

E01	PV above OVP.
E02	PV below UVL.
E04	OVP too low.
E06	UVL too high.
E07	OUT ON during fault.

# Driver Functional Description

This is the logical operation of the driver function.

……………………………………………………..

1. If serial port is closed,  return with an error.

2. If command is “ADR n”,  then wait 50 mSec.

3. Add checksum to command.

4. Loop up to four times,  or until PU gives successful response.

Send command, checksum and <CR> terminator.

Every 10 mSec,  read any characters returned by PU.
Do this until a <CR> is read to show the PU is finished sending.

If <CR> is not found in 500 mSec, then Loop again (go to step 4).

if <CR> is found before 500 mSec, then see if checksum in response matches one calculated for the response.
If checksum not match, then Loop again (go to step 4).

5. If command has been sent four times,  and there is still an error, 
    then return with an error.

6. Now have good response from PU.

7. If response is “C0n” or “E0n”,  then return with error.

8. Return showing SUCCESS and response message from PU.

End of driver function call.